ROBOVATOR Startup Problems

Note: while doing trouble shooting, it is important that you keep an eye on the battery voltage.

It should at least be 12 Volts

Use a multimeter to check that you do not drain the battery. You should connect a charger to the battery and disconnect the fuse for the fan. Pull the computers other than #1 and #3 unless you want to check all. Please do not skip a test or change the sequences. This manual has been carefully designed to identify the problem as quickly as possible

Problem: Display stuck at ”Please wait a minute” see how to check the display. If display is ok, check that the computer LED is ON. If ok, check the cables from wall socket of the machine to backplane #3. If cables are ok, swap computer #3 with another computer. If problem persists, check that the computer has booted, see how to

Problem: Display stuck at ”Syncing power controller”   This message means that there is a communication problem. It could be a problem with the network or some missing or corrupted files. see how to check the network. If the network is ok, check the system files on the USB.

If possible, use the backup USB. Make sure that the correct RSlicense file is present.

Look for files with a length of 0 bytes. (profile_0.ini normally has 0 bytes) but other files must not have a zero value.

Check that the LED on the startswitch is blinking at speed 2.  If not check the communication between computer #1 and the startswitch. See how to

How to check the network:

Verify that the Power LED of the computer #3 is ON. Do the ping test: use a laptop and connect the laptop with the machine using a network cable from the laptop to the network switch of the machine. Bring up the command window on the laptop and type: ping 192.168.1.113    and press “Enter”  if you get the message “destination host unreachable” it could mean:

1) the computer is not powered on or has a problem. Check that the power LED is  ON. If not, See how to check the power.

2) the cable from laptop to switch or from switch to computer #3 has a problem. Replace cables or verify cables on another network.

3) the switch has a problem. Replace the switch or check the switch by using all the ports

4) the computer has a problem

if you get the message “100% packets lost” it could mean:

1) the cable from laptop to switch or from switch to computer #3 has a problem

2) the switch has a problem. Replace the switch. Use a 10/100MB switch.

if you get a message containing “0% package loss” it means that this part of the network is ok.

Test the connection to computer #1 the same way using 192.168.1.110

Test the connection to the Router the same way using 192.168.1.1

How to check the display:

The display will say ”Please wait a minute” when it is powered on. This does not mean that the display communicates with the computer in slot #3. It will display this message even without any computers installed in the machine.

The best way to check the display is to put it on another machine to verify that it is working.

Power check:

The startswitch controls a relay that supply power the computers. The startswitch waits until the Router is up and running before it turns this relay ON. Seen how to check that the Router is up.

Startswitch communication check:

When power is ON the LED at the startswitch blinks slowly  at speed 1.

When the system starts and the computer  in slot #1 has booted it will try to talk with the startswitch. If successful, the LED starts to blink at speed 2. If not, computer has not booted or there is a problem with the cable between backplane #1 and the startswitch. See here on how to check if the computer #1 has booted. If computer has booted and the error persists, replace black telephone cable between “display” connector on backplane #1 and the startswitch. If problem persists replace small flat grey cable between “display” connector on IO-board and computer motherboard. If problem persists and all the above steps has been verified, replace startswitch.